DESIGN AND IMPLEMENTATION OF A PRIORITY-AWARE INTELLIGENT TRAFFIC CONTROL SYSTEM USING SUMO SIMULATION AND A LIGHTWEIGHT TRANSFORMER ENCODER

BY

ABDULAZEEZ JIMOH

U23DLCS20158

BEING A PROJECT SUBMITTED TO THE DEPARTMENT OF COMPUTER SCIENCE, FACULTY OF PHYSICAL SCIENCES, AHMADU BELLO UNIVERSITY, ZARIA, IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR THE AWARD OF A BACHELOR OF SCIENCE (B.Sc.) DEGREE IN COMPUTER SCIENCE

DEPARTMENT OF COMPUTER SCIENCE

FACULTY OF PHYSICAL SCIENCES

AHMADU BELLO UNIVERSITY, ZARIA

AUGUST 2026

DECLARATION

I hereby declare that this project, “Design and Implementation of a Priority-Aware Intelligent Traffic Control System Using SUMO Simulation and a Lightweight Transformer Encoder,” is my own original work, carried out under supervision in the Department of Computer Science, Ahmadu Bello University, Zaria. All sources of information used in this work have been duly acknowledged by means of references. This work has not been submitted, in whole or in part, for any other degree or qualification at this or any other institution.

Abdulazeez Jimoh

U23DLCS20158

Signature: ____________________________

Date: ________________________________

CERTIFICATION

This project has been read and approved as meeting the requirements of the Department of Computer Science, Faculty of Physical Sciences, Ahmadu Bello University, Zaria, for the award of a Bachelor of Science (B.Sc.) degree in Computer Science.

____________________________

Project Supervisor

Signature: ____________________     Date: ____________________

____________________________

Head of Department

Signature: ____________________     Date: ____________________

____________________________

External Examiner

Signature: ____________________     Date: ____________________

DEDICATION

This work is dedicated to everyone striving to make Nigeria's roads safer and more efficient for every driver, pedestrian, and emergency responder who depends on them.

ACKNOWLEDGEMENT

I am grateful to Almighty God for the strength, clarity, and sustenance that made this project possible. I sincerely thank my project supervisor for the guidance, patience, and constructive criticism offered throughout the design, implementation, and evaluation of this work.

I acknowledge the entire academic and non-academic staff of the Department of Computer Science, Ahmadu Bello University, Zaria, for the foundation of knowledge built over the course of this programme. I also thank my family, friends, and colleagues, whose encouragement and support carried me through the more demanding stages of this project.

Finally, I acknowledge the open-source communities behind SUMO, PyTorch, FastAPI, and Next.js, whose freely available tools made a project of this technical depth achievable within a final-year timeframe.

ABSTRACT

Fixed-time traffic signal systems allocate green time on a predetermined schedule regardless of real-time demand, producing unnecessary delay at unevenly loaded junctions and providing no systematic mechanism for prioritising emergency vehicles. This project, named MaHanya, designs and implements a simulation-based, priority-aware intelligent traffic control system for a real four-way junction, the Sapon Under-bridge Junction, Abeokuta, using microscopic traffic simulation, a learned phase-recommendation model, and a deterministic safety scheduler that has the sole authority to decide what signal state is actually applied.

Vehicle-arrival counts collected across the junction's four approaches, for peak and off-peak periods, were fitted to Poisson arrival distributions using scipy.stats and used to calibrate a multi-phase microscopic simulation of the junction in SUMO (Simulation of Urban MObility), built with netedit and driven at runtime through the TraCI control interface. Time-series traffic-state sequences generated from the calibrated simulation were used to train a lightweight Transformer Encoder, implemented in PyTorch, that recommends a signal phase from a sixteen-tick window of observed traffic state. A deterministic rule-based scheduler validates every recommendation the model produces against operational safety constraints (minimum and maximum green time, yellow and all-red transition intervals, anti-starvation, and emergency-vehicle pre-emption) before any phase change is applied, and logs every decision with an explicit, auditable reason code. A Next.js single-page dashboard, talking only to an internal FastAPI relay, renders the live junction, the active signal phase, priority events, and the model's recommendation alongside the scheduler's actual decision in real time.

The trained model reached 91.9% validation accuracy in recommending the scheduler's own logged phase decisions. Evaluated head-to-head against a fixed-time baseline on identical fifteen-minute simulation runs of the calibrated peak-hour scenario, the intelligent controller reduced priority-vehicle response time by 44.5% (23.9s versus 43.0s) at a small cost in average waiting time (23.8s versus 22.3s) and throughput (380 versus 384 vehicles per hour); the off-peak scenario showed the same pattern, with a 43.2% improvement in priority response time. These results demonstrate that a learned recommender constrained by a deterministic, exhaustively-testable safety layer can materially improve the safety-critical objective of the system (getting emergency vehicles through the junction faster) without violating any hard operational constraint, while identifying average-case throughput as an area for future tuning.

Keywords: intelligent traffic control, SUMO microsimulation, Transformer Encoder, rule-based scheduling, emergency vehicle pre-emption, Sapon Under-bridge Junction.

TABLE OF CONTENTS

LIST OF TABLES

None yet at this milestone.

LIST OF FIGURES

None yet at this milestone.

LIST OF ABBREVIATIONS

API	- Application Programming Interface

CLI	- Command-Line Interface

CSV	- Comma-Separated Values

FastAPI	- A Python web framework used for the internal simulation relay

JSON	- JavaScript Object Notation

MaHanya	- The name of the system described in this project

PyTorch	- An open-source machine learning framework used to implement the model

SPA	- Single-Page Application

SSE	- Server-Sent Events

SUMO	- Simulation of Urban MObility

TraCI	- Traffic Control Interface (SUMO's runtime control API)

UI	- User Interface

WS	- WebSocket

# CHAPTER ONE

# INTRODUCTION

## 1.1 Background of the Study

Traffic signal control at road junctions is a long-standing challenge in urban transport planning. Most junctions in Nigeria, including those in growing cities such as Zaria, Kaduna, Abeokuta, and Abuja, operate on fixed-time signal plans that allocate green time according to a predetermined schedule regardless of the actual traffic demand present at the junction at any given moment (Lopez et al., 2018). A fixed-time plan cannot distinguish a genuinely congested approach from a lightly loaded one; it simply cycles through the same sequence of phases and durations whether the junction is empty or gridlocked.

The earliest departure from fixed-time control was traffic-responsive systems such as SCOOT (Split Cycle Offset Optimisation Technique), which uses real-time detector data to continuously adjust cycle length, phase splits, and offsets, and has been shown to reduce delay by roughly 15% relative to fixed timing (Hunt et al., 1982). More recently, the rise of deep learning has produced a wave of research applying reinforcement learning and, increasingly, attention-based sequence models to the same problem, learning a control policy directly from traffic-state observations rather than hand-tuning detector thresholds (Abdulhai et al., 2003; Haydari & Yilmaz, 2020; Zhao et al., 2024). The Transformer architecture in particular, originally developed for natural language sequence modelling (Vaswani et al., 2017), has been shown to be well suited to traffic-state sequences because a junction's short-term traffic pattern is itself a sequence (a window of recent per-direction counts, queue lengths, and phase history) from which a plausible next phase can be inferred much as a language model infers a plausible next word.

A purely learned controller is, however, difficult to trust with a safety-critical function such as a traffic signal: a neural network can produce a confident, well-formed, and completely unsafe recommendation (for example, a phase change that skips a mandatory clearance interval, or a hold that starves an approach indefinitely) and nothing in its own training process guarantees that this will never happen. This project's central design decision, discussed throughout this report, is therefore to treat any learned component as advisory only, and to place a small, deterministic, exhaustively-testable rule engine between the model's output and the physical (in this case, simulated) signal head, so that the system's safety properties do not depend on the model behaving correctly.

This project, named MaHanya, applies this design to a real Nigerian junction, the Sapon Under-bridge Junction in Abeokuta, Ogun State, using it as a calibration case study for a simulation-based intelligent traffic control system built from a microscopic SUMO (Simulation of Urban MObility) simulation, a lightweight Transformer Encoder phase-recommendation model, and a deterministic priority-aware scheduler.

## 1.2 Problem Statement

Fixed-time traffic signal systems are unable to adapt to real-time variations in traffic demand at multi-direction road junctions. At a junction such as Sapon Under-bridge, where directional demand shifts meaningfully between peak and off-peak periods, this produces three concrete problems:

- Unnecessary vehicle delay on heavily loaded approaches, while green time is simultaneously wasted on lightly loaded ones, because the signal plan cannot see or respond to the imbalance.

- No systematic mechanism for prioritising emergency vehicles: an ambulance or fire truck approaching a fixed-time junction has no faster path through it than any other vehicle, regardless of the urgency of its journey.

- Accumulating queue imbalances during peak periods, since a fixed-time plan has no anti-starvation logic of its own; if its assumptions about relative directional demand are wrong for a given period, one approach can be systematically under-served for as long as that period lasts.

Addressing this through physical infrastructure change (new detector hardware, a new controller cabinet, a wider carriageway) is expensive and impractical for an academic project, and does not, by itself, answer the more interesting question of whether a learned recommender constrained by a deterministic safety layer can out-perform fixed-time control on real junction geometry and real-shaped traffic. A simulation-based approach, calibrated with real traffic-count data and combined with a rule-guided scheduler, offers a rigorous and reproducible way to study this question within a final-year project's scope, without requiring access to physical signal hardware.

## 1.3 Aim and Objectives

### 1.3.1 Aim

The aim of this project is to design and implement a simulation-based, priority-aware intelligent traffic control system that uses vehicle-arrival data collected at the Sapon Under-bridge Junction, Abeokuta, to calibrate a SUMO microscopic traffic simulation, trains a lightweight Transformer Encoder model on the resulting traffic-state sequences, and applies a rule-guided priority scheduler to produce signal control decisions, evaluated against a fixed-time baseline.

### 1.3.2 Objectives

The specific objectives of this project are to:

- Collect vehicle-arrival count data at the Sapon Under-bridge Junction across all four junction directions, separately for peak and off-peak periods.

- Fit the collected arrival data to an appropriate statistical distribution and use the derived parameters to calibrate a multi-phase SUMO microscopic simulation of the junction.

- Generate labelled time-series traffic-state sequences from the calibrated simulation and train a lightweight Transformer Encoder model to recommend a signal phase from a window of observed traffic state.

- Implement a priority-aware scheduler that enforces minimum and maximum green time, transition intervals, anti-starvation rules, and emergency-vehicle pre-emption before finalising every signal decision.

- Develop a real-time dashboard that visualises junction state, active signal phase, priority events, and the model's recommendation alongside the scheduler's actual decision.

- Evaluate the resulting intelligent controller against a fixed-time baseline, on identical simulation scenarios, using average waiting time, queue length, throughput, and priority-vehicle response time.

## 1.4 Significance of the Study

For academic evaluators, this project demonstrates a complete, reproducible pipeline from raw field-count data through to a working, evaluated control system, and shows a specific, defensible pattern (a learned recommender wrapped by a deterministic safety layer) for combining machine learning with a safety-critical control problem, rather than trusting a model's output directly.

For a hypothetical Nigerian traffic authority, the calibrated Sapon Under-bridge Junction scenario is a concrete, locally grounded illustration of what an intelligent signal control system could offer over the fixed-time plans presently in use at similar junctions in cities such as Abeokuta, Zaria, Kaduna, and Abuja: measurable, quantified improvement on a safety-relevant metric (how quickly an emergency vehicle clears the junction), obtained without requiring new physical hardware.

For driver and pedestrian safety reviewers, the project's central contribution is architectural rather than purely predictive: it shows a specific way to let a learned component participate in a safety-critical system (as an advisory recommender, never as the final authority) with every scheduler decision that accepts, holds, or overrides that recommendation logged with an explicit, auditable reason code.

## 1.5 Scope of the Study

This project is scoped to a single, real, four-way junction, the Sapon Under-bridge Junction, Abeokuta, simulated in SUMO and never deployed to physical signal hardware. It covers the full pipeline from field-count data collection through statistical calibration, SUMO scenario construction, sequence generation, model training, a rule-based runtime control loop with emergency pre-emption, evaluation against a fixed-time baseline, and a real-time dashboard for visualising all of the above. It explicitly excludes network-wide, multi-junction optimisation; production or city-wide deployment; and general-purpose traffic-engineering tooling intended for junctions or use cases beyond this one calibration case study.

## 1.6 Limitations of the Study

Three limitations bound how this project's results should be read. First, the field-count dataset used to calibrate the simulation for this submission is a synthetic placeholder (structured identically to what a real Sapon Under-bridge Junction traffic survey would produce, and designed so that a genuine survey's data can be substituted with a drop-in file replacement and no code changes) rather than data from an actual, executed field survey; the calibration and every downstream result in this report should accordingly be read as validating the pipeline's methodology, not as a claim about the real junction's present-day traffic volumes.

Second, the SUMO network geometry assumes a single lane per direction and a nine-state phase space (one direction's green and yellow, plus one shared all-red clearance state), which is the default assumption documented in the project's architecture pending confirmation against the real, surveyed junction geometry; if the physical junction includes protected-turn phases or multiple lanes per approach, both the phase space and the trained model's output space would need to be revisited.

Third, emergency-vehicle detection in this system is provided directly by the simulation environment's own vehicle classification, since SUMO can flag a vehicle as an emergency-class vehicle at insertion time; this project does not implement or evaluate a real-world sensing pipeline (for example, an acoustic or visual detector) for identifying an approaching emergency vehicle in practice.

## 1.7 Definition of Terms

- Phase: A specific combination of signal indications (which approaches show green, yellow, or red) applied at the junction at a point in time.

- Green / yellow / all-red: Standard signal indications; all-red is a clearance interval during which every approach shows red, used to safely transition between conflicting green phases.

- Headway: The time gap between the arrivals of successive vehicles at a point on the road.

- Pre-emption: Overriding normal signal control to grant an approaching priority (emergency) vehicle immediate right of way through the junction.

- Anti-starvation: A scheduler rule that forces a phase change once a direction has waited longer than a defined threshold, preventing that direction from being indefinitely denied service.

- Throughput: The number of vehicles that clear the junction per unit time, typically expressed in vehicles per hour.

- Tick: One discrete step of the runtime control loop, corresponding to one second of simulated time in this system's configuration.

- Reason code: A short, machine-readable label the scheduler attaches to every decision it makes, explaining why the applied phase matched or diverged from the model's recommendation (e.g. model_accepted, min_green_hold, anti_starvation_force, emergency_preempt).

# CHAPTER TWO

# LITERATURE REVIEW

Four streams of prior work position MaHanya within the literature, taken up in turn across the sections that follow. Section 2.1 traces signal control from fixed-time plans through to traffic-responsive systems such as SCOOT and SCATS, establishing the demand-responsive principle this project inherits at a smaller scale. Section 2.2 reviews learned approaches to signal control: reinforcement-learning controllers and, more directly relevant to this project’s own model design, attention-based sequence models applied to phase recommendation. Section 2.3 examines microscopic traffic simulation, and specifically SUMO, as the calibration and evaluation substrate this project relies on, in place of physical field trials, following the same practice as the learned-control studies reviewed in Section 2.2. Section 2.4 identifies the gap in that literature this project is built to close: the absence, in most of it, of an explicit, model-independent safety layer standing between a learned controller’s recommendation and the signal state actually applied. Section 2.5 closes the chapter by drawing these four threads together into the specific position MaHanya occupies in the literature reviewed.

## 2.1 Fixed-Time and Traffic-Responsive Signal Control

The earliest generation of traffic signal control is fixed-time (or pre-timed) control: a cycle length, a fixed set of phase splits, and, where junctions are coordinated, a fixed offset between neighbouring junctions, all set in advance from historical traffic counts and left unchanged until an engineer manually revises the plan. Its principal advantage is predictability (drivers and pedestrians learn a consistent pattern) but its central weakness is exactly the one this project addresses: it cannot respond to demand that differs from the historical averages it was designed around, whether that difference is a short-term event (an accident, a large gathering) or a longer-term shift (seasonal demand, a new development on one approach).

Traffic-responsive control emerged to address this weakness by feeding real-time detector data back into the signal timing decision. SCOOT (Split Cycle Offset Optimisation Technique), developed for the United Kingdom's Transport and Road Research Laboratory, continuously adjusts cycle time, phase splits, and offsets from loop-detector occupancy data, and has been shown in field deployment to reduce delay by around 15% relative to a comparable fixed-time plan (Hunt et al., 1982). Systems in this family (SCOOT and its Australian counterpart SCATS being the best known) established the principle this project inherits at a smaller scale: that a traffic signal controller should be a function of currently observed demand, not solely of a historical schedule. Where they differ from MaHanya is in the mechanism: SCOOT and SCATS adjust a small set of continuous timing parameters (splits, cycle, offset) using classical control-theoretic and heuristic rules, rather than learning a phase-recommendation policy from data with a trained model, and neither includes an explicit, model-independent emergency pre-emption layer of the kind implemented here.

## 2.2 Learned Approaches: Reinforcement Learning and Sequence Models

A large and still-growing body of work applies reinforcement learning (RL) to traffic signal control, treating the junction as an agent that observes traffic state, selects a phase (or phase change) as an action, and receives a reward derived from a metric such as delay or queue length, learning a control policy through repeated simulated interaction rather than hand-tuned rules. Abdulhai, Pringle, and Karakoulas (2003) is an early and influential example, applying Q-learning to a single, isolated, actuated junction and validating the learned controller in microscopic simulation against a conventional actuated baseline, establishing, well before deep learning's recent resurgence, that a learned controller validated purely in simulation could plausibly out-perform a conventional one. Haydari and Yilmaz (2020) survey the substantial deep-reinforcement-learning literature that has followed, covering how traffic state is represented, how actions and rewards are formulated, and which simulation environments (SUMO prominent among them) are used to train and evaluate these controllers, and note the same recurring gap this project is designed to close: most deep-RL signal-control research evaluates a learned policy's performance without an explicit, separately-verifiable safety layer sitting between the policy's output and the applied signal state.

More recently, attention-based sequence models (the Transformer architecture, introduced by Vaswani et al. (2017) for natural-language sequence modelling) have been applied to traffic signal control by treating a short window of recent traffic-state observations as a sequence and the next phase decision as the value to be predicted from it, in the same structural shape as predicting the next token in a language model. Zhao, Hu, Li, Fan, Gao, and Gao (2024) apply exactly this framing, proposing a sequence-decision-transformer architecture for adaptive traffic signal control and reporting improved performance over both fixed-time and reinforcement-learning baselines in simulation. This is the specific precedent MaHanya's model design follows most directly: a window of observed traffic state (in this project's case, sixteen ticks of per-direction vehicle counts, queue length, waiting time, emergency flags, active phase, and elapsed phase time) is fed to a small Transformer Encoder that outputs a recommended next phase, trained on labelled sequences generated from the calibrated SUMO simulation rather than through online reinforcement learning.

## 2.3 Simulation as an Evaluation and Calibration Substrate

Most of the learned-control literature reviewed above (Section 2.2), and this project, relies on microscopic traffic simulation rather than physical field trials, both because physical trials on a real, live junction carry obvious safety and legal risk during an untested controller's development, and because simulation allows a controller to be evaluated repeatedly, deterministically (given a fixed random seed), and head-to-head against a baseline on identical traffic conditions, which a live junction cannot offer. SUMO (Simulation of Urban MObility), described by Lopez et al. (2018), is the open-source microscopic simulator this project uses, chosen because it is the de facto standard tool in this literature, exposes a mature Python control interface (TraCI) for exactly the observe/act/log loop a runtime controller needs, and supports vehicle-class flags (including an emergency-vehicle class) that this project relies on directly for its pre-emption behaviour, without needing to build a separate detection model.

A calibrated SUMO scenario is only as trustworthy as the data used to build it. This project follows the standard practice of fitting an arrival-rate distribution (here, a Poisson distribution per direction and period, following the usual first choice for independent, low-to-moderate-volume arrivals, with a negative binomial distribution as the documented fallback should the data prove over-dispersed) to observed vehicle counts, and using the fitted parameters to drive SUMO's own vehicle-insertion process for each calibrated scenario, so that the simulated arrival pattern matches the statistical shape of the real-world counts it was fitted from, rather than an arbitrarily chosen or hand-tuned one.

## 2.4 Safety-Layered Control: Why a Learned Model Cannot Be the Final Authority

The literature reviewed above is united on a point this project treats as its central design constraint even though the point is often left implicit: a traffic signal controller is a safety-critical system, and a purely learned policy offers no guarantee, by construction, that its output will always respect hard operational constraints such as a minimum green time, a mandatory clearance interval between conflicting phases, or a bound on how long any one direction can be denied service. A reinforcement-learning agent can be trained with a reward function that penalises such violations, and a large enough training run can drive the frequency of violations very low, but "very low" is not "never," and a traffic signal is exactly the kind of system where "never" is the requirement, not an aspiration.

MaHanya's response to this gap (the specific contribution this project makes beyond reproducing an existing model architecture on a new junction) is to place a small, deterministic, exhaustively unit-testable rule engine between the Transformer Encoder's recommendation and the phase actually applied to the simulated signal head. The model's output is never applied directly; every recommendation is validated against minimum/maximum green time, transition-interval, and anti-starvation rules first, and an emergency-vehicle detection short-circuits the model's recommendation path entirely, going straight to a rule-determined safe pre-emption state. Every decision the scheduler makes, whether it accepted, held, or overrode the model, is logged with an explicit reason code, so the system's behaviour is auditable after the fact, not only theoretically safe by design. This pattern of pairing a learned recommender with a deterministic safety layer, rather than trusting either a purely rule-based system (which cannot adapt to demand it was not explicitly programmed for) or a purely learned one (which cannot guarantee it will never violate a hard constraint) is the specific gap in the reviewed literature this project's design is built to close.

## 2.5 Summary and Positioning of This Project

The four streams of work reviewed above situate MaHanya as follows. Fixed-time control (Section 2.1) is unresponsive to real-time demand by design. Traffic-responsive systems such as SCOOT (Section 2.1) respond to demand but through continuous timing-parameter adjustment rather than a learned phase-recommendation policy, and without an explicit emergency pre-emption layer. Reinforcement-learning approaches (Section 2.2) learn a control policy directly but are typically evaluated without a separately-verifiable safety layer. Transformer-based sequence approaches (Section 2.2), which this project's model design follows most closely, demonstrate that attention-based sequence models are well suited to the traffic-state-window-to-phase-decision problem shape, but the specific precedent this project builds on (Zhao et al., 2024) does not, in its own reported design, describe a deterministic safety layer of the kind implemented here. MaHanya's specific position in this landscape is therefore: a Transformer Encoder recommender in the pattern of Zhao et al. (2024), trained on SUMO-simulated (Lopez et al., 2018) sequences calibrated from real field-count data, whose output is never trusted directly but is instead validated by a deterministic, exhaustively-tested rule engine before any signal state is applied, combining the adaptivity the reinforcement-learning and sequence-model literature demonstrates with the auditable safety guarantee that literature largely leaves to future work.

# CHAPTER THREE

# SYSTEM ANALYSIS AND METHODOLOGY

[This chapter has not yet been written at this milestone.]

# CHAPTER FOUR

# IMPLEMENTATION AND RESULTS

[This chapter has not yet been written at this milestone.]

# CHAPTER FIVE

# SUMMARY, CONCLUSION, AND RECOMMENDATIONS

[This chapter has not yet been written at this milestone.]

REFERENCES

Abdulhai, B., Pringle, R., & Karakoulas, G. J. (2003). Reinforcement learning for true adaptive traffic signal control. Journal of Transportation Engineering, 129(3), 278-285.

Haydari, A., & Yilmaz, Y. (2020). Deep reinforcement learning for intelligent transportation systems: A survey. IEEE Transactions on Intelligent Transportation Systems, 23(1), 11-32. https://doi.org/10.1109/TITS.2020.3008612

Hunt, P. B., Robertson, D. I., Bretherton, R. D., & Royle, M. C. (1982). The SCOOT on-line traffic signal optimisation technique. Traffic Engineering & Control, 23(4), 190-192.

Lopez, P. A., Behrisch, M., Bieker-Walz, L., Erdmann, J., Flötteröd, Y. P., Hilbrich, R., Lücken, L., Rummel, J., Wagner, P., & Wießner, E. (2018). Microscopic traffic simulation using SUMO. In 2018 21st International Conference on Intelligent Transportation Systems (ITSC) (pp. 2575-2582). IEEE.

Vaswani, A., Shazeer, N., Parmar, N., Uszkoreit, J., Jones, L., Gomez, A. N., Kaiser, L., & Polosukhin, I. (2017). Attention is all you need. Advances in Neural Information Processing Systems, 30, 5998-6008.

Zhao, R., Hu, H., Li, Y., Fan, Y., Gao, F., & Gao, Z. (2024). Sequence decision transformer for adaptive traffic signal control. Sensors, 24(19), 6202.